# Worklog

I like to take notes as I work. These are things I observed and decisions I made. It is for my own sake of thinking-out-loud more than anything.

## Reading the files together

### 10/03

- Found that the devices had turned on and off as much as 26 times during the deployment. It seems that most of these are maybe the death-throws of the battery. In the big Cahuna data, there was 11 straight days and then 23 times of being turned on for a few seconds. Decided to delete any events that were less than 1 hour (because we couldn't get any meaningful data out of them anyway, really).  that solution worked.

- Not all GPS points are making it into the dataset (dropped about 20 points). Confirmed that all the GPS points fall inside the accelerometer range. Exploring the gps_data directly but found that sometimes the internal clock had tweaked and shown only a few seconds elapsed when the gps_timestamp showed that really a lot of time elapsed. See example image below.

  - This could easily have happened if there was corruption in the artemis writing to file (there’s a lot of corruption). Let’s go check the relevant file…

    - Attached below. Have highlighted the problem reads in yellow and their corresponding blue true reads. (Before and after this is fine btw.)

  - After a bit of thinking, the solution I can think of is to change the internal timestamp so it’s a relative change between the gps_timestamps. Because I don’t want to affect the other timestamps, I will just re-interpolate when the reads occur on the same internal_timestamp… this corrects as below:

    - It’s not perfect (note the only 2-minute increase between the previous point and the first of the highlighted). However, it is better. ##NOTE: Come back and improve this logic later…

- Went to pull the videos and they’re way off. My guess is that something has come in as AEST time rather than UTC, or has been converted somewhere. Will go back through carefully…

  - Yes, so the drone time is saving in local time rather than UTC. Therefore, I need to save it in as AEST and then convert to UTC before I pull from the data.

### 11/03

- I now have the metadata of when sampling started and stopped. Going to select only those days and then visualise the remainder to see what I’m working with.

- Here is an example of what it looks like visually:

  - From quick visual inspection, I can’t see any obvious issues. But nor am I seeing what I might expect in terms of big hops. I feel like we are seeing consistent sleep/inactivity periods though, which is a good sign.

- And yet, when I try to pull the correct section from the accel, it is just some random section of the data… HUH. There has to be some kind of time-zone issue. Maybe the timezone of the drone is not as expected? Apparently POSIXct coped with daylight savings automatically but one can never be too sure. We conducted this research in Fowler’s Gap, NSW, but are from Brisbane, QLD.

  - Sydney in winter: UTC+10 (AEST) (also the same as Brisbane)  No

  - Sydney in summer: UTC+11 (AEDT)  apparently this is already taken into account

  - Adelaide in winter: UTC+9:30 (ACST)  No

- While explaining this problem to Chris Clemente I realized that the issue might not actually be the timestamps of the drone but rather that the timestamps in the accel data are incorrect. (i.e., the problem I “dealt with” yesterday was actually insufficient. When showing him how the time-repeats worked, he explained how the boards worked on a hardware level and I realized that the issue was more significant than I had initially assumed. Therefore, we will be bringing Chris Bird (brilliant electrical engineer who built the boards in the first place) on to the project and get his help… Now creating debug reports from each of the kangaroos to give Chris tomorrow.

### 12/03

- Lola had no successful GPS hits and therefore we don’t have a matched time stamp. However, Chris B says we might be able to turn the garbled config statements back into information… This sounds really hard. Adding it to the #DOLATER list.

- I’m realizing that the internal timestamp being wrong is probably the issue

- Need to check whether the acceleromeyer also shows the problem, then it must be a board fault. If the accelerometer is normal, then it’s an issue with the serial communication between the board and the gos